WIND RESOURCE ASSESSMENT IN KARGIL

Wind Energy Development in Kargil has been initiated by the demonstration projects established by Kargil Renewable Energy Development Agency (KREDA) under the guidance of the NIWE, an R & D organisation established under the MNRE, in five most windy places of the district.

Five candidate sites in Kargil with 50m wind masts installed for wind resource assessment:

| S.No. | SITE NAME | BLOCK |
| --- | --- | --- |
| 01 | Khumbathang | TSG |
| 02 | Photang | Zanskar |
| 03 | Rangdum | Tai-Suru |
| 04 | Hambutingla | Kargil |
| 05 | Lakhthang | Drass |

The monitoring heights of the basic/optional parameters at all the five sites in District Kargil:

| S.No. | Parameters | Unit | Monitoring Heights |
| --- | --- | --- | --- |
| 01 | Wind Speed | m/s | 10m, 30m, 50m |
| 02 | Wind Direction | Degree | 30m, 50m |
| 03 | Temperature | (°C) | 03m |
| 04 | Pressure | Mb | 03m |
| 05 | Solar Radiation | w/m² | 03m |
|  | Note: Each site has same parameters at same heights. |  |  |

Wind Characteristics: The Average Wind Speed and Wind Density at various heights (Based on Oct 2011 to Aug 2012) for the five sites is given below.

| Station | 50 m |  | 30 m |  | 10 m |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WS | WPD | WS | WPD | WS | WPD |
|  | (m/s) | (W/m²) | (m/s) | (W/m²) | (m/s) | (W/m²) |
| Hambutingla | 4.07 | 90.09 | 4.05 | 96.19 | 4.41 | 115.92 |
| Lakhthang | 3.17 | 60.10 | 2.87 | 48.97 | 3.03 | 48.17 |
| Khumbathang | 3.03 | 67.65 | 2.79 | 60.35 | 2.77 | 46.12 |
| Rangdum | 3.08 | 48.16 | 3.09 | 44.21 | 2.91 | 35.81 |
| Photang | 4.80 | 161.15 | 4.58 | 144.69 | 3.90 | 86.86 |

It has been found that at most of the places in Kargil including Photang Zanskar, the wind nature is seasonal.

WS = Wind Speed.

WPD = Wind Power Density.